# Cervid Ballistics — Mathematical Models and Design Methodology

Technical Reference

August 2026

# Introduction

Cervid Ballistics is a single-file, offline-capable exterior ballistics calculator covering air rifle, rimfire, and centrefire disciplines. This document describes the mathematical models underpinning the trajectory engine and the engineering methodology that shaped the application’s design over its development history. It is intended as a technical reference for anyone reviewing, extending, or auditing the calculator’s physics or its software design decisions.

Two caveats apply throughout. First, several of the models below are widely used, well-established approximations rather than first-principles derivations — Miller’s stability formula and Litz’s spin-drift formula are the clearest examples — and the application’s own code comments are deliberately explicit about where this is the case. Second, all internal computation is carried out in imperial units (feet, seconds, grains, degrees Fahrenheit-derived Kelvin) regardless of the unit system the user has selected for display; metric values are converted only at the input and output boundary. This choice is discussed further in the design methodology section.

# Part 1 — Mathematical Models

## 1. Coordinate System and Numerical Integration

The trajectory engine integrates the equations of motion using a fourth-order Runge–Kutta (RK4) method, but — unlike most simple ballistic solvers, which step forward in time — it integrates with horizontal distance travelled (x, in feet) as the independent variable, not time. A fixed step of 3 feet (exactly one yard) is used throughout.

The state vector carried through the integration is:

- y — vertical position relative to the bore line (feet)

- v_x, v_y — horizontal and vertical velocity components (ft/s)

- t — elapsed time (seconds)

and the derivatives with respect to x are:

dy/dx  = v_y / v_x
dv_x/dx = −D / v
dv_y/dx = (−D·(v_y/v) − g_eff) / v_x
dt/dx  = 1 / v_x

where v = √(v_x² + v_y²) is the speed along the flight path, D is the drag deceleration (Section 2), and g_eff is gravitational acceleration after the Eötvös correction (Section 8).

Why integrate in x rather than t? Stepping in fixed 1-yard increments guarantees that every row of the range card — whichever interval the user has configured, in yards or metres — lands on an exact, physically computed sample point rather than an interpolated one. A time-stepped integrator would need to interpolate between two unevenly-spaced samples to recover the state at a clean range mark, introducing a small but avoidable extra source of error. It also made it straightforward to later add a “dense” sampling mode (one recorded point per yard, regardless of the table’s own interval) purely for smooth chart rendering, without touching the underlying physics at all — the same integration loop just records more of the points it was already computing.

The classical RK4 update, applied at each step, is:

k1 = f(state)
k2 = f(state + (dx/2)·k1)
k3 = f(state + (dx/2)·k2)
k4 = f(state + dx·k3)
state_next = state + (dx/6)·(k1 + 2k2 + 2k3 + k4)

applied simultaneously across all four state variables (y, v_x, v_y, t).

## 2. Aerodynamic Drag

Deceleration along the flight path is computed from the standard point-mass exterior-ballistics drag equation:

D = ρ_ratio × K × Cd(Mach) × v² / BC

where ρ_ratio is the air density relative to the standard reference atmosphere (Section 6), K = 2.085 × 10⁻⁴ is a fixed empirical constant, Cd(Mach) is the drag coefficient at the current Mach number, v is true airspeed, and BC is the projectile’s ballistic coefficient (in the same drag-model standard, G1/G7/GA, as the Cd table being used). Because BC already normalises for the reference projectile’s own geometry under whichever standard is selected, the equation needs no separate diameter, mass, or cross-sectional-area terms.

Cd(Mach) is obtained by linear interpolation between adjacent points of a fixed reference table, selected according to the drag model the user has chosen:

| Model | Reference shape | Notes |
| --- | --- | --- |
| G1 | Flat-base, blunt “generic” projectile | The oldest and most widely published standard; most manufacturer-quoted BCs are G1 |
| G7 | Boat-tail, streamlined long-range profile | A closer match to modern low-drag rifle bullets; produces a flatter, more realistic transonic drag curve for those shapes |
| GA | Diabolo airgun pellet | A community-sourced table (matching the data embedded in ChairGun/Strelok Pro) built specifically because G1 and G7 — both shaped around rifle-bullet geometry — fit diabolo pellets poorly, particularly the sharp transonic drag rise pellets show above roughly Mach 0.8. Treated as the best available reference rather than an authoritative published standard |

Each table stores [Mach, Cd] pairs; values below the lowest or above the highest tabulated Mach number are clamped to the nearest endpoint rather than extrapolated.

## 3. Ballistic Coefficient Standards (G1 ↔ G7)

A G1 BC and a G7 BC for the same physical projectile are not the same number — each normalises drag against a differently shaped reference standard, so the two are not interchangeable. The application always requires a G1 BC. A G7 BC is optional: if the user supplies a genuine published G7 figure, it is used directly whenever the G7 drag model is active. If it is left blank, the model falls back to an estimate:

BC(G7, estimated) = BC(G1) / 1.9

The ratio 1.9 is a commonly cited approximation for modern boat-tail rifle bullets; it has no general validity for pellets or flat-base projectiles, and the interface flags the value as an estimate whenever it is in use rather than presenting it as an entered figure.

## 4. Zeroing (Sight-In) Solver

Before the visible trajectory is computed, the engine has to solve for the vertical launch angle (relative to the sight line) that makes the projectile cross the sight line exactly at the user’s chosen zero distance — the same problem a shooter solves by adjusting turrets at the range.

This is done in two stages:

- Time-to-zero pass. A short simulation at zero launch angle establishes t₀, the time of flight to the zero distance.

- Iterative angle refinement. An initial vertical velocity offset is estimated analytically —

v_yB = (scopeHeight/12 + zeroDistance·tan(angle) + ½·g·t₀²) / t₀ − MV·sin(angle)

— then refined by up to eight passes of a damped correction loop: each pass re-simulates the shot with the current candidate offset, measures the vertical gap between point of impact and true sight line at the zero distance, and nudges the offset by 85% of the gap (scaled by time) before trying again. The loop exits early once the residual gap is under 0.005 inch. This is a simplified damped fixed-point solver, not a formal bisection or Newton–Raphson method — it converges quickly in practice because the relationship between launch angle and point of impact is close to linear over the small angles involved.

## 5. Wind Deflection

Wind input uses a clock-face convention: 3 o’clock is a full-value crosswind from the shooter’s right, 9 o’clock a full-value crosswind from the left, and 12/6 o’clock represent a pure head/tailwind with no crosswind component. The crosswind speed actually used by the model is:

W(ft/s) = windSpeed(mph) × 1.46667 × sin(clockPosition/6 × π)

Deflection is then computed using the classic “lag rule”: a bullet drifts downwind by an amount proportional to how much longer it has actually spent in flight than a hypothetical drag-free bullet travelling in a straight line at muzzle velocity would have taken to cover the same distance —

drift(in) = W × (t_actual − x/MV) × 12

Documented simplification: only the crosswind component of a wind at any clock angle is modelled; a head/tailwind component (which would otherwise slightly speed up or retard the projectile along its own line of flight) is not simulated separately.

## 6. Atmospheric Density Correction

Drag scales with the actual air density relative to a standard reference atmosphere (15 °C, 1013.25 hPa). The density ratio is computed from user-entered altitude, temperature, humidity, and station pressure using the standard humid-air correction:

Psat = 611.21 × exp[(18.678 − T/234.5) × (T/(257.14+T))]      (Arden Buck equation, T in °C)
Pv   = (RH% / 100) × Psat                                       (water-vapour partial pressure)
Pd   = P − Pv                                                    (dry-air partial pressure)
ρ_ratio = (Pd / P_std) × (T_std / T_K) × (1 − 0.378 × Pv/P)

The final (1 − 0.378·Pv/P) term corrects for humid air being less dense than dry air at the same temperature and pressure (water vapour has a lower molar mass than the nitrogen/oxygen mix it displaces) — a refinement many simplified ballistic calculators skip in favour of treating humidity as negligible.

## 7. Speed of Sound and Mach Number

a(ft/s) = 1116.45 × √((T°C + 273.15) / 288.15)

an ideal-gas approximation referenced to standard sea-level temperature, used to convert true airspeed to Mach number for the drag-table lookup at every integration step.

## 8. Coriolis Effect and the Eötvös Correction

For long shots, the Earth’s rotation measurably deflects the trajectory. The model uses Earth’s mean angular rotation rate, Ω = 7.292115 × 10⁻⁵ rad/s, combined with the shooter’s latitude (φ) and the azimuth of the shot relative to true north (A), split into its two classically-distinguished components:

Eötvös effect (vertical). The bullet’s motion across the rotating Earth modifies its apparent gravity depending on whether it is travelling with or against the direction of Earth’s spin:

g_eff = g − 2Ω·cos(φ)·sin(A) × v_x

so an eastward-fired shot experiences very slightly reduced effective gravity (marginally less drop) and a westward shot the opposite, applied continuously through the integration rather than as an end-of-flight correction.

Horizontal Coriolis drift. A second, independent deflection is accumulated perpendicular to the line of fire, from the horizontal component of the Coriolis acceleration:

a_lateral = 2Ω·(sin(φ)·v_x − cos(φ)·cos(A)·v_y)

integrated twice (acceleration → velocity → displacement) alongside the main trajectory, so it correctly compounds over the whole time of flight rather than being approximated from a single average velocity.

At typical air-rifle and even most rimfire/centrefire ranges these terms are extremely small; they become practically relevant mainly at longer centrefire distances, which is why latitude and azimuth are exposed as explicit, optional inputs rather than defaulted silently.

The underlying magnitude and formula above are unchanged, but the results table presents this figure, like Wind Deflection and Spin Drift, in whichever unit — inches, cm, MOA, or mils — the table’s own display toggle is set to, converted with the same angular-conversion formulas as Section 11, rather than being fixed to linear inches or cm regardless of the table’s other units.

## 9. Spin Drift

Spin-stabilised projectiles drift slightly to the side of the twist direction over the course of a flight, independent of any crosswind. The model uses Litz’s approximate spin-drift formula (from Applied Ballistics for Long Range Shooting):

SpinDrift(in) = twistDirection × 1.25 × (Sg + 1.2) × t²

where Sg is the gyroscopic stability factor (Section 10) and t is time of flight in seconds. Being a function of t², spin drift grows disproportionately at long range even though it is negligible at short range — consistent with real-world long-range shooting data.

## 10. Gyroscopic Stability

Whether a spinning projectile is adequately stabilised is estimated with Miller’s stability formula:

Sg = 30·W / (n² · d³ · l_c · (1 + l_c²) · ρ_ratio)

where W is projectile weight in grains, d is diameter in inches, n is the twist rate expressed in calibers (twist length ÷ diameter), and l_c is the length-to-diameter ratio. The same relationship is solved in reverse to report the minimum twist rate required for a given projectile:

T_min = d × √(30·W·ρ_ratio / (1.5·d³·l_c·(1+l_c²)))

Greenhill’s rule is also available as a simpler, independent cross-check:

T = C × d² / l         (C = 150 below 2700 fps muzzle velocity, 180 at or above it)

Shape correction factors. Miller’s formula implicitly assumes a roughly solid, uniform-density projectile, so it has no way to account for a boat-tail shedding mass toward the rear, or a diabolo pellet’s mostly-hollow skirt — both of which need less spin than the formula would otherwise predict for the same length-to-diameter ratio. The application applies commonly-cited, community-sourced correction factors on top of the raw Miller result — explicitly documented in the source as approximate corrections, not a first-principles recalculation of the projectile’s actual moment of inertia:

| Shape | Sg multiplier | Twist-requirement multiplier |
| --- | --- | --- |
| Flat-base | ×1.00 | ×1.00 |
| Boat-tail | ×1.08 | ×√1.08 |
| Diabolo (airgun pellet) | ×1.20 | ×√1.20 |

The resulting Sg is classified into bands for display: below 1.0 Unstable, 1.0–1.4 Marginal, 1.4–2.5 Stable, 2.5–4.0 Over-stabilised, above 4.0 Excessively fast.

A known, documented gap: Miller’s formula (and the shape factors above) carry no muzzle-velocity correction term, despite being referenced to roughly 2800 fps rifle velocities — while air-rifle loads typically run 700–950 fps. Adding the commonly suggested (V/2800)^(1/3) velocity term has been deliberately deferred pending validation against pellet data that already reads plausibly without it, rather than being added speculatively.

## 11. Energy and Angular-Unit Conversions

Kinetic energy at any point in the flight:

KE(ft·lb) = v² × (W_gr / 7000) / (2 × 32.174)
KE(J)     = KE(ft·lb) × 1.35582

using 7000 grains per pound and g = 32.174 ft/s² as the weight-to-mass conversion, and the standard ft·lb-to-joule constant.

Linear drop and drift are converted to angular holdover units using the standard small-angle approximations, with a cos²(shot angle) correction applied to project the true (sloped) range onto the horizontal — the same simplification commonly known as “rifleman’s rule”:

MOA  = −drop(in) × cos²(angle) / (range(yd)/100 × 1.0472)
Mils = −drop(in) × cos²(angle) / (range(yd) × 0.036)

(1.0472 in per 100 yd defines one MOA; 0.036 in per yd defines one mil — both standard reference constants.)

## 12. Trajectory Truing (Empirical BC Calibration)

Manufacturer-quoted BCs are averages and frequently do not match a specific rifle/load/chronograph combination exactly. Trajectory Truing lets a shooter correct for this using one real, observed data point: a known range and the holdover (in mils, MOA, inches, or cm) they actually needed to hold at that range.

The engine solves for a single BC multiplier by binary search over the range [0.3, 2.5] (up to 40 iterations, convergence tolerance 0.0005 units in whichever unit the observation was entered in):

- A higher BC means less drag and therefore less drop, so a smaller predicted holdover value.

- If the model currently predicts more holdover than observed, BC is too low and is nudged up; if it predicts less, BC is too high and is nudged down.

- If the observation falls outside what any BC in the search range could produce, the multiplier is clamped to whichever bound gets closest, rather than continuing to search fruitlessly.

The resulting multiplier is applied to the base BC for the remainder of that session (and can optionally be saved back into the load’s preset), effectively calibrating “how much this specific bullet actually decelerates” from one field observation rather than trusting the catalogue BC in isolation.

A separate, deliberately non-destructive Verification step lets the shooter check that calibration against a second, independent real observation at a different distance, without re-fitting anything. It reports the model’s prediction, the observed value, the difference, and a qualitative verdict (holding up well / some drift / diverging), the last of which is framed as a prompt to re-check muzzle velocity or another input — since a genuinely good BC fit should generalise across distances, and a growing gap downrange usually points to a different input being wrong rather than the BC itself.

## 13. Zero Offset (Manual Holdover Correction)

A shooter who has zeroed a rifle for one load, then switches to a different projectile without re-zeroing, often already knows from field experience how much that new load holds over or under point of aim at the zero range. Zero Offset lets that known figure be entered directly — in mils or MOA — and applied to the whole table, without touching the drag model at all.

It is a deliberately distinct correction from Trajectory Truing above, not a variant of it. Truing solves for a BC multiplier so the drag model matches one observed data point, which implicitly assumes the zero itself is a true, zero-drop reference. Zero Offset does the opposite: it takes the zero as a starting point that is deliberately not a true zero, and shifts the entire table by a constant angular amount to match a directly known holdover.

The offset is converted once, on entry, to a single canonical mils-equivalent value, then applied as a term added to the drop calculation at every range:

drop(in) -= offsetMils × range(yd) × 0.036 / cos²(angle)

Because mils and MOA are angular units, a constant angular holdover corresponds to a growing linear correction downrange rather than a fixed number of inches — the term above scales with range for exactly that reason, and divides through cos²(angle) so an inclined shot’s holdover is not distorted by the same slope correction already applied to drop (Section 11). Since the correction modifies the shared drop value every other output is derived from, the results table, chart, CSV export, Range Card view, and turret-strip graphics all reflect it automatically with no separate implementation in each; energy, velocity, windage, and spin-drift columns are untouched, since the correction is a pure holdover shift rather than a change to the physical model.

## 14. Plausibility Checks

Before every calculation, entered values are checked against soft, per-discipline bounds (air rifle / rimfire / centrefire) on muzzle velocity, projectile weight, and BC (0.01–1.0 for all disciplines, the realistic range for small-arms projectiles generally). These checks are deliberately advisory only — they surface a dismissible warning (e.g. “muzzle velocity looks unusual — check fps vs m/s”) to catch likely typos or unit mix-ups, but never block or alter a calculation. This reflects a considered design choice, discussed further in Part 2.

# Part 2 — Design Methodology

## 1. Single-File, Zero-Dependency Architecture

The entire application — markup, styling, and logic — is one self-contained HTML file with no build step, no server component, and no external runtime dependencies. This was a deliberate, foundational constraint rather than an accident of scope: it guarantees the calculator works fully offline (relevant in the field, away from signal), opens instantly in any browser including Safari on macOS and iOS, and can be copied, backed up, or shared as a single file with no installation process. Every feature added over the project’s life had to fit within that constraint, which in turn shaped several other decisions below.

## 2. Canonical Internal Units, Converted Only at the Boundary

All physics computation is carried out in imperial units internally, regardless of whether the user has the interface set to imperial or metric display. Metric values are converted to imperial on input and back to metric only for display. This was chosen over maintaining two parallel unit-aware code paths through the physics engine, which would have doubled the surface area for bugs and made the two systems drift out of agreement over time.

This decision was validated by a real bug it prevented from being worse than it was: an early metric-interval implementation rounded each successive table row’s native interval mark (e.g. every 10 m) to the nearest yard independently by repeatedly adding a fixed yard increment, which let small rounding errors compound row over row into a visible drift (…80 m, 91 m, 101 m, 111 m…). The fix — generating each target mark directly in its native unit and converting each one independently to the nearest yard, rather than accumulating an already-rounded step — is a direct consequence of the “single canonical unit, convert at the boundary” philosophy: the bug was in the conversion boundary, not the physics, and was fixed there without touching the engine at all.

## 3. Iterative, Usage-Driven Feature Development

Features were added in direct response to concrete friction encountered in real use, rather than speculative up-front design. Trajectory Truing and its later Verification step, the Range Card view, live-weather auto-fill, preset drag-reordering (including a dedicated touch-based reimplementation once it became clear the native HTML5 drag API silently does nothing for ordinary elements on iOS Safari), and the recent overflow-menu consolidation of per-preset actions are all examples of a feature being introduced, used, found lacking in some specific way, and then refined — sometimes across several successive passes — rather than being designed exhaustively before first use.

Compare Loads is the most extensive recent example of this pattern: shipped first with a fixed, auto-computed reference distance and an explicit Compare button, then made editable on request, then extended to read a currently-loaded preset’s live, unsaved amendments rather than always the stored version, and finally refined again once real use showed one of its warnings was firing too often to stay useful. See Part 3.

## 4. Testing Without a Browser: Headless Node/VM Harnesses

The development environment has no access to a real browser or to the npm package registry, which rules out both conventional browser-automation testing and most off-the-shelf JavaScript testing frameworks. In their place, a custom pattern was developed: the application’s actual <script> block is loaded verbatim into a Node vm context, against a small hand-built fake DOM (fake elements supporting classList, dataset, closest(), event listeners, and enough of the canvas 2D context API to record what a chart would have drawn) and a fake localStorage. This allows the real, unmodified production functions — computeTrajectory, renderPresets, drawChart, and so on — to be exercised directly and their outputs asserted on, without reimplementing or mocking the logic under test.

Every fix or new feature is paired with a test that specifically exercises the bug or behaviour in question — not a superficial smoke test — and existing harnesses are re-run after every change to catch regressions. Several bugs were caught specifically because a harness was built to reproduce the exact reported symptom (for example, simulating a touch landing on a nested SVG icon rather than its parent button, which a naive same-element check would miss) rather than a generic version of the problem.

## 5. Multi-Variant Deployment Discipline

The calculator exists as four synchronised files: the full-featured main application, a stripped-down “General Release” public build with no built-in preset library, and two deployment mirror copies (a site-root index.html and a personal-folder copy) kept byte-identical to their source in every shared code path. Every change to shared logic — the preset system, the chart, the physics engine — is applied identically across all four and reverified in each before being considered complete, rather than treating one file as canonical and letting the others drift.

## 6. Transparency About Approximations

Several of the physics relationships used are well-established approximations rather than exact derivations, and this is treated as something to document plainly rather than obscure behind confident-looking output. The G1↔G7 BC estimate ratio, the shape-based stability correction factors, and the community-sourced airgun drag table are all flagged in code comments as approximations with a stated provenance and stated limits on where they’re valid — and the project maintains an explicit, honestly-worded “known gaps” list (for example, the missing muzzle-velocity correction term in the stability calculation) rather than presenting the current model as complete.

## 7. Advisory Validation, Not Gatekeeping

Where the application checks user input for plausibility — velocity, weight, BC — it always does so as a dismissible, non-blocking warning rather than a hard validation error. The design preference throughout is to help a user catch their own likely mistake (a decimal point, an fps/m-s mix-up) without ever preventing them from running a calculation on genuinely unusual but real inputs, such as an intentionally low-powered air rifle load or an experimental wildcat cartridge.

## 8. Accessibility as a First-Class Concern

Distinguishing information is never conveyed by colour alone. On the trajectory chart, the drop and energy curves are differentiated by both colour and line style (solid versus dashed) so the chart remains legible to colourblind users; the same principle was applied when auditing other colour-only status indicators elsewhere in the interface. This was addressed as a dedicated pass over the application rather than an incidental side-effect of a colour palette choice.

## 9. Deliberate Simplicity Over Feature Completeness

Not every capability that was built was kept. A Bluetooth integration with a Kestrel 5700X weather meter was implemented and then deliberately removed once two problems became clear: Web Bluetooth is Chrome-only, directly conflicting with the project’s core Safari/iOS compatibility requirement, and the meter’s proprietary wind-speed encoding could not be reliably decoded. Rather than persisting with a partially-working, platform-incompatible feature, it was replaced with a simpler browser-geolocation-plus-weather-API “live weather” button that works identically everywhere the application itself runs. This stands as the clearest example of the project consistently preferring a smaller feature that works everywhere over a larger one that only works some of the time.

## 10. The Changelog as an Engineering Discipline

Every change, however small, is recorded in a dated changelog entry written in prose: what changed, why it was needed, and how it was verified. This is treated as part of the engineering process itself rather than after-the-fact documentation — it is what makes the “transparency about approximations” and “usage-driven iteration” principles above actually auditable after the fact, rather than asserted.

# Part 3 — The Compare Loads Feature

## 1. Purpose and Architecture

Compare Loads lets a shooter select up to three saved presets — any mix of built-in and custom, and any mix of the three disciplines — and see headline stats and an overlaid trajectory chart for all of them side by side, motivated directly by a real use case: deciding which of several rifle/load combinations to bring to a competition.

The feature required no changes to the trajectory engine itself. Because computeTrajectory() is a pure function of a self-contained params object with no dependency on the live form, a params object can be assembled for any saved preset and passed through the same, unmodified engine used for the single-load Results page. Comparison is therefore an assembly-and-presentation problem — building the right params object for each selected load and rendering three results side by side — rather than a second calculation path that risks drifting out of agreement with the first.

## 2. Reference Distance Resolution

All compared loads are shown out to one shared distance, so their chart lines end together and every headline figure is read at the same range. By default this is the shortest of the selected loads’ own max ranges — the furthest distance every selected load can be shown at without extrapolating any of them — but the distance is also directly editable, defaulting to that auto-suggested figure and re-suggesting it as the selection changes, until the shooter types a value of their own, at which point the typed value is left alone across further selection changes and only resets to auto-suggest once the selection is cleared back to zero.

Recomputation is explicit rather than live: neither changing the selection nor editing the distance field recalculates anything by itself, only the next press of the Compare button — consistent with the single-load page’s own Calculate Trajectory model, and avoiding a rapid sequence of recomputations while a shooter is still deciding what to compare.

The distance is not capped to any load’s own max range; the engine will extrapolate to whatever value is typed in. Instead, a load shown beyond its own saved (or, for a live-matched load, currently configured — see Section 3) max range triggers a dismissible, non-blocking warning rather than a hard limit, consistent with the advisory-only validation philosophy established in Part 2, Section 7.

## 3. Live vs. Saved State Resolution

A selected preset may also be the one currently open on the single-load Inputs tab, possibly with amendments that have not yet been saved back to the preset — a BC from Trajectory Truing, an edited Zero Offset, a changed weight. Compare resolves this by identity rather than by trying to detect whether something has changed: a selected slot is treated as “live” only when its mode, preset type, and name all match the preset currently loaded on the Inputs tab and a calculation has actually been run for it — the same name-based identity match already used elsewhere in the application for the baked-truing badge.

When a slot matches, Compare clones the exact params object already driving the live Results page — rather than rebuilding one from the stored preset — so any unsaved amendment is picked up automatically, with the shared reference distance substituted in. The clone is never written back to either the live form state or the saved preset; Compare only ever reads. When a slot does not match — a different preset selected, nothing loaded yet, or the mode has since been switched — it falls back to building a fresh params object from the stored preset, exactly as it would if the live-matching logic did not exist at all. No separate “has this been amended” detection was needed: reading live state whenever the identity matches produces identical figures to the saved preset whenever nothing has actually changed, so the fallback behaviour falls out automatically rather than being a distinct code path.

| Condition | Params source | Shown as |
| --- | --- | --- |
| Selected preset is loaded on Inputs, with a calculation already run | Cloned from the live results (lastResults.params) | “(live)” |
| Selected preset is not currently loaded, or nothing has been calculated yet | Rebuilt fresh from the stored preset | Preset name only |
| Mode has been switched since a preset was loaded | Rebuilt fresh from the stored preset (live-match state is cleared on mode switch) | Preset name only |

## 4. Centrefire-Scoped Subsonic-Crossing Warning

An earlier version of Compare flagged any load that crossed into subsonic flight before the comparison distance, using the same per-row isSubsonic flag the single-load Results table already computes. In practice this fired on a large share of comparisons in the air rifle and rimfire disciplines, where subsonic flight is the ordinary case — airgun pellets and slugs, and standard-velocity rimfire ammunition, are subsonic as a matter of course — making the warning noise rather than signal for exactly the disciplines where it fired most often.

The warning is now gated to centrefire selections only. Subsonic flight in a centrefire load is usually a deliberate choice rather than the default — a purpose-built subsonic .300 BLK load for a suppressed rifle is the clearest example — so a warning there is more likely to be catching something genuinely worth a second look. The gate is a single mode check against the selected preset’s own recorded discipline, evaluated per selected load rather than globally, so a mixed comparison of a centrefire and an air-rifle load correctly warns for one and stays silent for the other in the same comparison. This mirrors the project’s wider preference, discussed in Part 2, Sections 6 and 7, for validation that reflects what is actually unusual for the discipline in question rather than a single blanket rule applied everywhere.

## 5. Testing Methodology

Compare Loads is exercised with the same headless Node/vm harness pattern described in Part 2, Section 4, extended to simulate full real-world interaction sequences rather than only individual helper functions — selecting a preset, editing a field, calculating, opening Compare, and pressing Compare again, in that exact order — since the live-matching logic in particular depends on state built up across several successive user actions rather than any single function call.

One deployment variant, the General Release build, intentionally ships with no built-in preset library, which meant the comparison test suite could not assume any built-in preset indices exist. Tests were written to seed their own custom presets directly, via the same save path a real user would go through, making the suite portable across all four synchronised files regardless of which one, if any, ships built-in data — consistent with the multi-variant deployment discipline described in Part 2, Section 5.

# Appendix — Key Constants Reference

| Constant | Value | Used for |
| --- | --- | --- |
| g (gravity) | 32.174 ft/s² | Trajectory integration, energy calculation |
| Ω (Earth’s rotation rate) | 7.292115 × 10⁻⁵ rad/s | Coriolis / Eötvös effect |
| Standard temperature | 288.15 K (15 °C) | Air density ratio, speed of sound |
| Standard pressure | 101325 Pa (1013.25 hPa) | Air density ratio |
| BC standard ratio (G1→G7 estimate) | 1.9 | Estimating BC(G7) when not entered |
| Spin drift coefficient | 1.25 | Litz spin-drift formula |
| Shape factor — flat-base | ×1.00 | Stability (Sg) correction |
| Shape factor — boat-tail | ×1.08 | Stability (Sg) correction |
| Shape factor — diabolo pellet | ×1.20 | Stability (Sg) correction |
| Greenhill constant (MV < 2700 fps) | 150 | Alternative twist-rate rule |
| Greenhill constant (MV ≥ 2700 fps) | 180 | Alternative twist-rate rule |
| 1 MOA | 1.0472 in @ 100 yd | Angular holdover conversion |
| 1 mil | 0.036 in @ 1 yd (3.6 in @ 100 yd) | Angular holdover conversion |
| ft·lb → J | × 1.35582 | Energy display conversion |
| Grains per pound | 7000 | Energy calculation mass conversion |
| Integration step | 3 ft (1 yd) | RK4 trajectory integration |
| Trajectory Truing search range | BC multiplier 0.3–2.5 | Empirical BC calibration |